COMPARATIVE STUDY ON the Antioxidant Activity of EXTRA VIRGIN OLIVE OIL SAMPLES USING a Newly Developed Electrochemical Method and DPPH Spectrophotometric ASSAY

# Irina-Georgiana Bulgaru (Munteanu), Constantin Apetrei

## 1- Department of Chemistry, Physics and Environment, Faculty of Sciences and Environment, “Dunărea de Jos” University of Galaţi, 47 Domneasca Street, 800008 Galaţi, Romania

## georgiana.munteanu@ugal.ro, apetreic@ugal.ro

Background: Olive oil, which is the main source of dietary fat, is a complex mixture of natural compounds having multiple beneficial effects on the body. The phenolic content of olive oil confers to this product antioxidant, free radical scavenging, cardioprotective, and anticancer properties.

Aim: The present study describes the development of an electrochemical method based on a screen-printed carbon electrode modified with carbonaceous nanomaterials used for the determination of antioxidant activity of several extra virgin olive oil samples. At the same time a correlation between the data obtained by the proposed electrochemical method and those obtained by applying DPPH (2,2ʹ-diphenyl-1-picrylhidrazyl) spectrophotometric method has been performed.

Method: The optimised method based on cyclic voltammetry was used to determine the antioxidant properties of extra virgin olive oil samples and the results were compared and correlated with the results obtained by DPPH spectrophotometric assay.

Results and discussion: The cyclic voltammetry (CV) measurements were carried out in the hydroalcoholic extract of the extra virgin olive oil samples and in solutions of catechin of different concentrations. The analysis of the radical scavenging properties of the real samples and catechin solutions by DPPH method were correlated with the electrochemical data expressed as the area under the anodic branch of the cyclic voltammogram. The CV method results were in good agreement with the results obtained by the spectrophotometric method.

Conclusion: The results obtained from this study indicate that the extra virgin olive oil extracts possess antioxidant properties and could serve as free radical inhibitors or scavenger or, acting possibly as primary antioxidants. Moreover, the developed method could represent a valuable alternative for the analysis of antioxidants in complex samples than the classical spectrophotometric method that is prone to interference from colour and turbidity of the samples and also usually requires pre-treatment of the complex samples.

Acknowledgments. This work was supported by a grant of the Romanian Ministry of Education and Research,

CNCS—UEFISCDI, project number PN‐III‐P4‐ID‐PCE‐2020‐0923, within PNCDI III.

Keywords: olive oil, antioxidant activity, cyclic voltammetry, DPPH spectrophotometric assay