ZrCu thin film metallic glasses as biomaterials for coating of metallic orthopedic implants

# Gabriela A. Juravlea1,2, Alina Vladescu1, Anca C.Parau1

## 1- National Institute of Research and Development for Optoelectronics - INOE 2000, 409 Atomistilor Street, R077125, Magurele, Romania

## 2- University Politehnica of Bucharest, 313 Independeței Street, Bucharest, Romania

## gabriela.juravlea@inoe.ro

Abstract: In the current study, ZrCu-based thin film metallic glasses were developed as biomaterials especially for coatings of metallic orthopedic implants. The following coatings were proposed: ZrCu containing small amounts of Si, Ca, Sr, Mo and Mg. The films were deposited with a high deposition rate by a cathodic arc technique. Mostly utilized for orthopedic implants, Ti6Al4V alloy was used as substrates.

EDS and XRD techniques were used to analyze the elemental and phase compositions of the films. Scratch tests and surface profilometry were used to examine hardness, adhesion, and roughness. The investigation of the film corrosion resistance played an important role. The corrosion rate was calculated after these experiments, who were carried out in SBF at 37oC. For all of the tests, uncoated alloys were used as a reference sample. After electrochemical tests, the morphology of the generated films was examined.

All of the films enhanced the qualities of the uncoated substrates, according to the tests. The electrochemical behavior revealed that by adding Si, Mo, and Sr to the ZrCu structure, it improves the corrosion rate of uncoated and coated substrates significantly.

Acknowledgements: We acknowledge the support of the Romanian Ministry of Education and Research, CNCS - UEFISCDI, project PN-III-P4-ID-PCE-2020-1264 (PCE95/2021), within PNCDI III, and Romanian National Core Program no.PN18N-01-02/2019.

Keywords: metallic glasses, thin films, corrosion