ELECTROCHEMICAL DETERMINATION OF HYDROXYTYROSOL IN EXTRAVIRGIN OLIVE OILS USING SCREEN-PRINTED ELECTRODES MODIFIED WITH SINGLE WALL CARBON NANOTUBES AND TYROSINASE

# Alexandra Virginia Bounegru, Constantin Apetrei

## ”Dunarea de Jos” University of Galati, Faculty of Sciences and Environment

## 111 Domneasca Street, 800201, Galați, Romania

## e-mail address alexandra.meresescu@yahoo.com, apetreic@ugal.ro

Background: Olea europaea L., commonly known as olive, is a small tree species distributed mainly in the Mediterranean countries, with the main derivative product being olive oil [1]. Olive oil is present in the Mediterranean diet, as it is considered a protective factor for the prevention of chronic diseases, due to its antioxidant, anti-inflammatory, lipid-lowering and even antimicrobial action [2].

Aim: The aim of this study is to develop a reliable, sensitive and selective tyrosinase-based biosensor (SWCNT-Ty-SPE) for the determination of hydroxytyrosol (HT) and its quantification from real samples of extra virgin olive oil (EVOO).

Method: Tyrosinase (Ty) will be immobilized by casting and crosslinking, after prior modification of a screen-printed carbon electrode (C-SPE) with single-walled carbon nanotube (SWCNT-SPE). The characterization of the biosensor surface was performed both by electrochemical methods (CV, DPV, EIS) and by spectrometric methods (FTIR). The calibration curves of HT was performed by cyclic voltammetry (CV). Quantitative determinations were carried out from various samples of EVOO using CV.

Results and discussion: The presence and the sensitive properties of SWCNT and Ty were highlighted by all applied electrochemical methods (CV, DPV, EIS). SWCNT-Ty-SPE have increased selectivity and sensitivity, confirming the biocatalytic activity of Ty immobilized on the biosensor surface. Following the application of several scanning rates (0.1-1.0 V/s), it can be stated that, in all cases, the reduction process is controlled by HT adsorption on the active surface, which is faster and more obvious in the case of the biosensor. Based on the superior sensing properties of the biosensor, the calibration curve was subsequently realized in a wide concentration range of 0.1 M - 300M, with linearity in the range of 0.5-15 M and the limit of detection and quantification in the micromolar range. Subsequently, SWCNT-Ty-SPE was succesfully used to quantify HT from real EVOO samples, using CV.

Conclusion: In conclusion, the new biosensor developed based on SWCNT and Ty has multiple advantages such as: sensitivity, selectivity, feasibility and low cost. SWCNT-Ty-SPE could also be used in routine analyzes for food quality control, such as vegetable oils containing HT.

Acknowledgments. This work was supported by a grant of the Romanian Ministry of Education and Research,

CNCS—UEFISCDI, project number PN‐III‐P4‐ID‐PCE‐2020‐0923, within PNCDI III.

Keywords: hydroxytyrosol, tyrosinase, biosensor, olive oil

Reference

1. 	Foscolou, A.; Critselis, E.; Panagiotakos, D. Olive Oil Consumption and Human Health: A Narrative Review. Maturitas 2018, 118, 60–66, doi:10.1016/j.maturitas.2018.10.013.

2. 	Silva, R.M.A. da Evaluation of the Antimicrobial Potential of Natural Extracts on Helicobacter Pylori. In Proceedings of the Evaluation of the antimicrobial potential of natural extracts on Helicobacter pylori; December 9 2020.